ATM edge switch supporting live-to-air streaming

- CBR and UBR services to support AES47

- throughput: 3.6 Gbit/s non-blocking

- routes different sample rates simultaneously

- LEDs indicate when live-to-air traffic is flowing

RouteLive switch specifications

Front panel connectors

- 100 Mbit/s ports (A1-A16 on RL01-312 pictured above)

- RJ45, standard Ethernet pinout

- auto-MDIX (no cross-over cables needed)

- automatically selects between loop-through to Ethernet network and ATM-over-Ethernet service

- 155 Mbit/s ATM ports (B1-B8)

- RJ45

- pins 1,2,7,8 ATM data

- pins 4,5 “emergency AES3”

- pins 3,6 timing reference signal output

- 622 Mbit/s ATM ports (C1-C2)

- fibre, SFP modules

- transmission distance: 0.5, 2, 15, or 40 km

- All network ports

- two tri-colour LEDs per port

- LEDs show which ports are live-to-air

- “Emergency AES3” ports

- RJ45

- one pair connects to each of four ATM ports

- direct connection, effective even when power off

- pins 4,5 connect to port 1 or 5

- pins 1,2 connect to port 2 or 6

- pins 3,6 connect to port 3 or 7

- pins 7,8 connect to port 4 or 8

- Timing reference

- RJ45 input carries up to four separate copies

- signal from pins 3,6 distributed to ATM ports

- separate buffer amplifier for each ATM port

- other pairs routed to RJ45 “out” connector

- single cable from main distribution amp can feed up to 4 switches by daisy-chaining “out” to “in”

Back panel connectors and LEDs

- Two RS232 serial ports

- 9-way D-plug connectors

- baud rates up to 115,200

- Status indication

- two tri-colour LEDs

- shows whether the switch is live-to-air

- Mains power inlet

- switched and fused IEC connector

- 85-265V AC, 47-440Hz

Mechanical

- 19” rack mount unit

- 1U high

- 350mm deep

Configuration options

- RL01-112

- 16 ports 155 Mbit/s ATM over UTP

- 2 ports 622 Mbit/s ATM over fibre

- RL01-312

- 16 ports “Brunhilde” ATM service over 100 Mbit/s Ethernet physical layer

- 8 ports 155 Mbit/s ATM over UTP

- 2 ports 622 Mbit/s ATM over fibre

ATM services

- UNI 3.0, 3.1, and 4.0

- ILMI 4.0

- Additional messages defined by ATMAVA to support live-to-air traffic (proposed future IEC 62379 standard)

- CBR and UBR SVCs

RouteLive extension box specifications

Back panel connectors

- Two mains power inlets

- fused IEC connector

- 90-264V AC, 47-63Hz

- 25-way D-socket

- link to switch (cable supplied)

- supplies 24v power to switch

- connects switch processor to display etc

Front panel

- 2 lines × 40 characters LCD display provides menu-driven access to status information for the switch and calls passing through it

- 5-key keypad for navigating menus

- “power on” LED for each mains supply

- “battery” LED indicates state of charge

- “status” LEDs repeat the back panel “status” LEDs on the switch

- “reset” switch recessed behind panel

Mechanical

- 19” rack mount unit

- 1U high

- 350mm deep

Variation to switch specification

- Mains inlet replaced by 25-way D-socket for link from extension box

Switch management

PC software

- Windows program supplied

- connects via RS232 port

- can manage up to 128 units (switches and other equipment) at a time via the network

- supports ATMAVA control interface specification (proposed future IEC 62379 standard)

Switch reboot

- takes less than 5 seconds under normal circumstances

- can restart processor without tearing down ATM calls

Nine Tiles Networks Ltd

9, Station Road,

Waterbeach,

Cambridge CB5 9HT

Tel: +44 1223 862599

http://www.ninetiles.com